IoT-based Soldier Tracking and Health Monitoring System

Arduino Coding

#include <Wire.h>

#include <TinyGPS++.h>

#include <LiquidCrystal_I2C.h>

// === Pin Definitions ===

#define TEMP_SENSOR_PIN 4

#define HEARTBEAT_SENSOR_PIN 36

#define DANGER_SWITCH 32

// === Serial Ports ===

HardwareSerial gpsSerial(1); // GPS: RX=26, TX=27

HardwareSerial gsmSerial(2); // GSM: RX=16, TX=17

TinyGPSPlus gps;

LiquidCrystal_I2C lcd(0x27, 16, 2);

// === BPM Calculation Variables ===

unsigned long lastBeatTime = 0;

unsigned long bpmCalcStart = 0;

int beatCount = 0;

int bpm = 0;

void sendSMS(String message) {

gsmSerial.println("AT+CMGF=1");

delay(1000);

gsmSerial.println("AT+CMGS=\"+91 6385345859\"");

delay(1000);

gsmSerial.print(message);

delay(500);

gsmSerial.write(26);

delay(3000);

}

void setup() {

Serial.begin(115200);

gpsSerial.begin(9600, SERIAL_8N1, 26, 27); // GPS on UART1

gsmSerial.begin(9600, SERIAL_8N1, 16, 17); // GSM on UART2

pinMode(TEMP_SENSOR_PIN, INPUT);

pinMode(HEARTBEAT_SENSOR_PIN, INPUT);

pinMode(DANGER_SWITCH, INPUT_PULLUP);

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print(" Soldier Monitor ");

delay(2000);

lcd.clear();

bpmCalcStart = millis();

}

void loop() {

unsigned long currentTime = millis();

// === Temperature Reading ===

int tempValue = analogRead(TEMP_SENSOR_PIN);

float voltage = (tempValue * 3.3) / 4095.0;

float temperature = voltage * 100.0;

// === Heartbeat Signal ===

int signal = analogRead(HEARTBEAT_SENSOR_PIN);

if (signal > 2000 && (currentTime - lastBeatTime) > 300) {

beatCount++;

lastBeatTime = currentTime;

}

// === GPS Parsing ===

while (gpsSerial.available()) {

gps.encode(gpsSerial.read());

}

float latitude = gps.location.isValid() ? gps.location.lat() : 0.0;

float longitude = gps.location.isValid() ? gps.location.lng() : 0.0;

// === BPM Calculation Every 15s ===

if (currentTime - bpmCalcStart >= 15000) {

bpm = beatCount * 4; // 15s window

beatCount = 0;

bpmCalcStart = currentTime;

// === Always show values on LCD ===

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("T:");

lcd.print(temperature, 1);

lcd.print("C H:");

lcd.print(bpm);

lcd.setCursor(0, 1);

lcd.print("Lat:");

lcd.print(latitude, 1);

lcd.print(" Lon:");

lcd.print(longitude, 1);

delay(3000);

// === Check for abnormal values ===

bool tempAbnormal = (temperature < 34.0 || temperature > 40.0);

bool bpmAbnormal = (bpm < 60 || bpm > 100);

if (tempAbnormal && bpmAbnormal) {

String msg = "Heart & Temp Abnormal\nTemp: ";

msg += String(temperature, 1);

msg += "C, Heart: ";

msg += String(bpm);

msg += " BPM\nLocation: https://maps.google.com/?q=";

msg += String(latitude, 6) + "," + String(longitude, 6);

sendSMS(msg);

} else if (tempAbnormal) {

String msg = "Temp Abnormal: ";

msg += String(temperature, 1);

msg += "C\nLocation: https://maps.google.com/?q=";

msg += String(latitude, 6) + "," + String(longitude, 6);

sendSMS(msg);

} else if (bpmAbnormal) {

String msg = "Heart Abnormal: ";

msg += String(bpm);

msg += " BPM\nLocation: https://maps.google.com/?q=";

msg += String(latitude, 6) + "," + String(longitude, 6);

sendSMS(msg);

}

}

// === Emergency Switch Pressed ===

if (digitalRead(DANGER_SWITCH) == LOW) {

String alert = "EMERGENCY!\nLocation: https://maps.google.com/?q=";

alert += String(latitude, 6) + "," + String(longitude, 6);

sendSMS(alert);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("!! ALERT SENT !!");

delay(5000);

}

delay(100); // Fast loop

}